This assignment may be done by groups of at most two (2) students per group.

The objective of the final assignment is to model and design a controller for the SegwayTM Human Transporter. A detailed description of this device may be found at http://travel.howstuffworks.com/ginger.htm.

- Derive the system’s equations of motion (system of two 2nd order differential equations) relating the frame angular displacement () and linear velocity v to the input torque u in terms of the constant parameters of the system (e.g., masses and moment of inertias) shown in the figure.

- Linearize the equations of motion in (a) about =0 and show that the transfer functions from v and u and from  to v is of the following forms:

V/U=a(s2-b)/[s(s2-c)] and /U=ds/(s2-b)

Where a,b,c, and d are positive constant coefficients that depend on the system parameters. Find the numerical values for these coefficients using a set of realistic numerical values for the system parameters.

- The control objective is to regulate the linear velocity v and balance the frame at the same time. To achieve this objective, the following control law is suggested:

U(s)=Kv(vd-v)-C(s)(s)

Where Kv is the velocity gain, vd is the desired speed, and C(s) is the balancing controller to be determined. First, find Kv to regulate the speed ignoring the frame rotation (i.e., V/U=ab/sc) with a closed-loop time constant of T=0.02 second.